Private Pre-Shared Key Sync with Google Workspace Groups Guide

by Tim Smith, SA – 06/15/2026 – v3.0.1

Overview:

This guide covers setting up and running the script to sync your Google Workspace users with the Private Pre-shared-key (PPSK) within ExtremeCloud IQ (XIQ) public cloud only. PPSK is a solution provided by Extreme Networks to fill the gap between a Wi-Fi SSID solution using a single PSK for all users and deploying a complete 802.1X solution. Extreme Networks PPSK solution allows creating a dedicated key for each user or device on the identical SSID, lowering the number of SSIDs broadcasting in the air and minimizing the airtime consumption due to overhead management frames. This solution also adds the ability to assign VLANs based on user/device groups to avoid the need for separate SSIDs to segregate these groups.

This guide enables you to leverage your existing Google Workspace User groups to automatically create a Private Pre-shared key for every Google Workspace user and remove the PPSK user if a user is disabled or removed from the group in Google Workspace.

Each Google Workspace user must have a unique email address for this script to work correctly.

Target Audience:  Technical

# PPSK Use Cases:

- Identity for IoT devices

- BYOD for employees

- Staff device onboarding

- Secure Guest Onboarding (time-based keys with employee sponsorship)

- Hospitality vertical using the hyper-segmentation feature, Private Client Groups (PCGs)

- Third-party via API integration

Table of Contents

Overview	1

Target Audience	1

PPSK Use Cases:	1

Prerequisites:	4

Scripting Environment Preparation:	4

Information:	4

Device Choice:	4

Python Installation:	5

5

Mac OSX Sonoma	5

Windows 11	5

Required Modules:	5

Checking for existing Modules	5

Installing required modules	5

6

Script Variables:	6

PCG Support (optional)	7

Generating the XIQ Token	7

Swagger	7

Login	8

Authorize in Swagger	8

Generating Specific Tokens	10

Creating the Google Workspace Token and Refresh Token	11

Creating the Google Workspace Project	11

11

Creating the Google Workspace API OAuth Consent Screen	12

Creating the Google Workspace API OAuth Credentials	12

Generating a Token and Refresh token	14

Google Workspace Group Name	15

XIQ User Group ID	15

XIQ Network Policy ID	16

Running the Script:	16

Log File	17

Scheduling Script to Run	17

Mac & Linux-based Systems	17

Setting up a Cron Job	17

Cron Job Time Format	18

Cron Job Script and Script Location	18

Cron Job Output and Job Completion	18

Cron Job Command Example	19

Windows based Systems	19

Setting up Windows Task Scheduler	19

Start a Program	19

Editing the Time	20

Troubleshooting:	21

Log File	21

Invalid XIQ token	21

Invalid XIQ token format	21

Expired XIQ token – Code 401 & JWT expired	21

Invalid XIQ Username/password – Code 401	21

Failed Authentication for Google Workspace API	21

Invalid or Wrong Group Name entered	21

XIQ User Failed to Create – Code 400	22

XIQ Timeout Error	22

Gsuite_test.py	22

# Prerequisites:

- ExtremeCloud IQ Public Cloud, Private Cloud (IQVA on-prem is not supported.)
o The key directory can be stored in the cloud (unlimited keys) or local on all access points (10,000 key maximum limit)

- Knowledge of XIQ by adding access points, creating network policies, and SSIDs

- XIQ PPSK SSID and associated User Groups configured

- RadSec Proxy requires TCP Port 2083 to be open on your internet firewall

- One or more XIQ native access points

- Not supported on wired systems, A3 NAC, or campus-based Wi-Fi systems (WiNG or IdentiFi)

- Download the following files

  - XIQ-GSuite-PPSK-Sync.py

    - Version 3.0.1 is the current version. See lines 10-15 in the script

  - Gsuite_setup.py

    - Version 1.1.0 is the current version. See lines 10-15 in the script

    - Used to generate the token and refresh token – see the generate token and refresh token section.

  - Gsuite_test.py (optional – see troubleshooting section)

    - Version 1.1.0 is the current version. Lines 11-16

  - requirements.txt (optional – see modules section)

  - app folder – containing the following required scripts

    - gsuite.py – handles all Google Workspace API calls

    - logger.py – handles logging functions for both console output and log file

    - xiq_api.py – handles all XIQ API calls

# Scripting Environment Preparation:

### Information:

The XIQ-GSuite-PPSK-Sync.py script requires, at minimum, Python 3.6 and tested up to Python 3.13. This script can be executed manually but ideally would be set up as a cronjob to be run every 8, 12, or 24 hours. This script can be executed from any device with Python and the needed modules installed. This device must reach the Google Workspace Cloud and reach to ExtremeCloud IQ.

When run, the script will create a XIQ-GSuite-PPSK-sync.log file in a script_log folder inside of the app folder.  This log file will show information about PPSK users created and deleted. It will also show how many users were parsed from XIQ and the Google Workspace user group when run. Any API errors experienced will also show up in the log file.

## Device Choice:

This script can be executed from any device running Python 3.6 or higher. The device could be a RedHat server, a pc/laptop running Windows 11 or Mac OSX, or even a Raspberry Pi-type device. The device will need to be on the network and be able to reach the Google Workspace Cloud as well as reach ExtremeCloud IQ. This can be done through a proxy. Proxy config is beyond the scope of this guide.

## Python Installation:

Depending on the device used, you may need to install Python or a different version of Python. The easiest way to check the version of Python is to open the terminal (Power Shell on Windows) and type this command.

python3 --version

Below are some examples of installing python3 for Windows and Mac OSX. Linux systems that were tested all had python3.6 or higher installed by default.

### Mac OSX Tahoe

- Open the terminal and enter python3 –version

  - This triggers the installation of Developer Tools

- Click Install

- Click Agree

- The Developer tools that installed python3 will also install pip3 in Sonoma

- Mac terminal will be used to install python modules

### Windows 11

- Search Microsoft Store for Python 3.12 and click install

- Log in with Microsoft credentials

- The Windows store installs pip3 with python3. Pip3 will be used to install the needed modules.

## Required Modules:

The requests, google-api-python-client, google-auth-httplib2, google-auth-oauthlib

modules are the only module required for the XIQ-GSuite-PPSK-Sync.py script. For Linux and Mac

### Checking for existing Modules

python3 -c “import requests”

You can check if the required module is installed using the terminal (PowerShell for Windows). Run the following command.

The module is not installed if a ‘ModuleNotFoundError: No module named '<module name>' error is returned.

### Installing required modules

The required module can be installed using pip3 using the downloaded requirements.txt file with the following command.

pip3 install requests

pip3 install -r requirements.txt

pip3 install google-api-python-client

pip3 install google-auth-oauthlib

pip3 install google-auth-httplib2

Or the module can be installed individually using.

# Script Variables:

The Global Variable section of the script (file name: XIQ-GSuite-PPSK-Sync.py) must be updated with the correct values. We will briefly cover each of these and, for some, will go into more detail below.

Line 21 is info for Google Workspace

- gs_domain – The Customer’s domain name in Google Workspace.

Line 24 is the number of PPSK and PCG users to be returned per API call.

Lines 26 – 30 are for XIQ authentication. 2 methods could be used, but the token method is recommended.

- XIQ username and password –

  - Lines 26 and 27 - uncomment by deleting the # at the beginning of the line. Then fill in the username and password.

  - Line 30 - comment out the line by adding # to the beginning of the line.

- Token method – a token can be generated to only allow access to view/create/delete PPSK users. This is the preferred method. Details on generating this token are below in the Generating the XIQ Token, specifically the Generating Specific Tokens sub-section.

Lines 32 – 36 are where we will define which Google Workspace groups to sync with which XIQ User Groups. Note that the brackets around the groups create a list. Each set of groups should be in a set of parentheses; a comma should separate each element, and a coma should separate each set of groups. If only one set of groups is required, line 35 can be deleted, and the comma at the end of line 34 removed.

- We will cover how to get the needed Google Workspace group name in the Google Workspace Group Name section.

- We will cover how to get the XIQ User Group ID in the XIQ User Group ID section.

- NOTE: The order is very important here. If the same Google Workspace user is in multiple groups, the user will be put in the first XIQ User Group in the list. XIQ users can only be in one PPSK User group.

#### PCG Support (optional)

With PCG enabled, the script will take longer to run.

The API call for PCG user creation supports batching and long-running operations, meaning the API will kick off a job to create the users and then check its completion. The script will do this in batches of 100 users. The script will periodically check the job until it completes and then move on to the next batch.

The API call for PCG user deletion supports batching but not LRO. So, the script will attempt to delete 100 PCG users at a time. If the total count of PCG users doesn’t change, or XIQ responses with a 504 Gateway Time out error, the script will retry the API call. The script will attempt up to 10 times before skipping. If batch delete is skipped there will be errors when the script tries to delete the associated PPSK user.

Line 38 To enable PCG Support, change the PCG_Enable Variable from False to True.

Lines 40-46 If PCG is Enabled, PCG_Mapping should be updated with the correct information. If PCG is not Enabled, PCG_Mapping will not be used and does not need to be updated.

- Line 41 – This should be replaced with the XIQ User Group ID number that correlates with the PCG. See XIQ User Group ID section.

- Line 42 – This is the name of the User Group associated with the ID on line 42

  - This is needed to add and remove Users from the PCG.

- Line 43 – This is the Network Policy ID associated with the PCG

  - We will cover how to get the Network Policy ID in the Network Policy ID section

- Line 44 – This is the Network Policy Name associated with the PCG

Line 49 – When this is True, the PPKS user name will combine the Google display name and email address. – <displayName_email> This allows the script to work when multiple Google users that have the same display name with different email addresses.

## Generating the XIQ Token

You can view our developer portal site at https://developer.extremecloudiq.com/ There is a link to our swagger page here and other developer tools. There is also a Communities section to reach out to with any questions.

#### Swagger

We will use the swagger interface to generate the token https://api.extremecloudiq.com/.
On the swagger page, clicking on any API will expand information about the API and allow you to try it. Clicking the “Try it out” button, filling out any needed information, and then clicking the execute button will allow you to try that specific API call.

The 2nd generation APIs are based on access tokens generated by a XIQ account. External XIQ accounts need to be granted API to leverage the new APIs. See A Guide to Getting Started with v2 APIs in XIQ for more details. Currently, these tokens can only be generated through the /login POST API request. They cannot be generated through the XIQ GUI.

##### Login

Request Body

{
  "username": "xiq@example.com",
  "password": "changeme"
}

The /login POST request is used to generate an access token. In the request body, enter a local administrator XIQ account username and password, and the API will respond with an access token that can be used for any following calls. This token will be valid for 24 hours after creation, and this token will have the ability to be used for any of the API calls the user is authorized for within XIQ.

For this script, we will use this token to generate a separate token with limited access and a specified expiration time. Copy the access token created, not including the “”s.

Authorize in Swagger

At the top of the Swagger page, click the authorize button. A window will pop up, allowing you to paste the access token. Clicking “Authorize” Swagger will set Swagger to use the added access token for the API calls on the page.

##### Generating Specific Tokens

The /auth/apitoken POST request allows you to specify an expiration time and set permissions for a token. This is a great way to create a token for a specific application or script, only allowing the token to perform the needed tasks.

The expiration time uses Epoch time, which is the number of seconds since midnight on Jan 1, 1970 (UTC). https://www.epochconverter.com/ is a webpage that can convert a readable time to an epoch time or epoch time to a more readable time. Set a time for 1 year out and get the epoch time.

For this script, we will want to have the following permissions - enduser, pcg:key, lro 
This will give us access to view, create, and delete ppsk users and view, create, and delete pcg-key-based users if necessary.

Adding the desired expiration time and a list of permissions, this API will return a token only usable by the specified APIs.

Request Body

{
  "description": "Token for XIQ-AD-PPSK-Sync.py  script",
  "expire_time": 1628186428,
  "permissions": [
  "enduser”,
  “pcg:key”,

“lro”
  ]
}

Response Body

{

"access_token": "eyJhbGciOiJIUzI1NiJ9.eyJzdWIiOiJ0aW1qc21pdGgyNEBwcm90b25tYWlsLmNvbSIsInNjb3BlcyI6WyJhY2NvdW50OnIiXSwidXNlcklkIjoyMTc5MjMyMSwicm9sZSI6IkFkbWluaXN0cmF0b3IiLCJjdXN0b21lcklkIjoyMTc5MTk3MSwiY3VzdG9tZXJNb2RlIjowLCJoaXFFbmFibGVkIjpmYWxzZSwib3duZXJJZCI6MTc5MTYxLCJvcmdJZCI6MCwiZGF0YUNlbnRlciI6IklBX0dDUCIsImlzcyI6ImV4dHJlbWVjbG91ZGlxLmNvbSIsImlhdCI6MTYyODE4MzA4OSwiZXhwIjoxNjI4MTg2NDI4fQ.CtBGq4YVGB9FzCodr6Oi5IG8yy1-4B-77AWl5rVG3S0",

"create_time": "2021-11-29T15:47:57.000+0000",

"expire_time": "2021-11-29T16:10:08.000+0000",

"creator_id": 21792321,

"customer_id": 21791971,

"description": "Token for XIQ-Gsuite-PPSK-Sync.py script",

"permissions": [

"enduser",

"pcg:key",

“lro”

]

}

Copy the newly created access_token and add it to the XIQ_token variable in the script.

## Creating the Google Workspace Token and Refresh Token

A token must be generated every time the script is run to access the Google Workspace user data using API. This token is only valid for 1 hour but can be easily generated within the script using a provided client_secret.json file and a Refresh token. The following sections below cover what is needed to allow API access to your Google Workspace environment, create the client_secret.json file, authorize the script to use the JSON file information and generate the refresh token.

##### Creating the Google Workspace Project

To create the Google Workspace Project needed to access the Google Workspace user data using API, open a browser and go to console.cloud.google.com. Once logged in, click the dropdown on the top left. This could have the domain name you are using, another project you’ve created, or you may say “New Project.” Clicking that dropdown will open a new window allowing you to select different projects you have. A button to create a new project is on the top right of that window.

In the newly opened window, set the project name you will create. You can optionally edit the Project ID as well if you want. Once completed, click the Create button.

The project will take a second to create. You can monitor the notification section to see once it's complete.

Once you see the green check in the notifications, you can open the project. You can do this by clicking Select Project in the notification or clicking the dropdown and selecting the new project.

Open the APIs & Services tab on the left with the new project selected.

We will first need to enable the Admin SDK API. This will allow API access to groups and users.

Search for “Admin SDK API” and select the result. This will open a window where you can choose to Enable the APIs.

##### Creating the Google Workspace API OAuth Consent Screen

Now that your Google Workspace project is created, you must set up the Consent Screen for OAuth. This will allow you to provide approval for the script to be granted access. This page will just be used when running the Gsuite_setup.py script. See the Generating Google Workspace token and refresh token section.

On the left of the APIs & Services page, select the OAuth consent screen tab and click Get started.

For the App information, fill out the App name, select the user support email. Click Next.

Click Internal, then Next.

Enter an email for the Developer contact information. Click Next.

Check the agreement, and click Continue.

Lastly click Create.

We will be setting the scope for these using

the Gsuite_setup.py script.

##### Creating the Google Workspace API OAuth Credentials

Next, we must create the OAuth credentials for the Google Workspace API.

Select the Clients tab on the left of the APIs & Services page. At the top of that page, choose Create client.

When the OAuth client ID opens, select the Desktop app for the Application Type.

Then on the next window, enter the name you would like for the client and click Create.

Select the ‘Download JSON’ button when the OAuth client is created.

The JSON file will download to your browser's default location. The file will be named something like “client_secret_{the_id_created}.apps.googleusercontent.com.json”. Rename this file to “client_secret.json” and move it to the folder the scripts are located.

##### Generating a Token and Refresh token

Using the client_secret.json file, the Gsuite_setup.py script will allow you to authorize the scripts to access your Google Workspace user data. Ensure the client_secret.json file has been renamed and added to the same folder as the scripts. You can run the Gsuite_setup.py script in a terminal by going to the location of the scripts and running:

Python3 Gsuite_setup.py

When this script runs, a URL will be provided for you to authorize access. A web browser may open on this page as well.

With the linked browser page open, select the Google Workspace user.

On the next page presented, confirm the Project name. You can also see what permissions are requested from the script. As you can see, they are all view only.

Once validated, select Allow.

You can then close the browser history, and the script will complete. A new file will be created called gsuite_token.json. This file has the needed token and refresh token info, the client id, and the client secret. This file will be used in the Gsuite_test.py and the XIQ-GSuite-PPSK-Sync.py scripts to generate a new token to be used in those scripts.

## Google Workspace Group Name

Each group in Google Workspace will have a unique name assigned to it. An easy way to get this name is to log into your Google Workspace Console, go into Google Workspace Directory on the left, and select Groups. You can see all the groups with the Group Name listed in the column.

Once the group name is obtained, add it to the group_roles object in the script.

group_roles = [

# Google Workspace GROUP Name, XIQ group ID

("PPSK Users Group", "XIQ group ID"),

("Google Workspace GROUP Name", "XIQ group ID")

]

## XIQ User Group ID

Each XIQ User Group will be assigned a unique ID when created. This is something that the backend systems use and is not seen in the GUI. The easiest way to get the ID is from the Swagger page.

Return to the swagger page, scroll to the Configuration – User Management section, and find the /usergroups GET request. 
Click the “Try it out” button, then the “Execute” button. When you find the Name of the XIQ User Group you want to use, it will be inside a pair of {curly brackets}. Inside the same pair of curly brackets will be an element called id, and this is the ID that is needed.

Response Body

{

"page": 1,

"count": 10,

"data": [

{

"id": 769490635824436,

"name": "Home_Hive",

"description": "",

"predefined": false,

"create_time": "2021-10-11T18:24:33.000+0000",

"update_time": "2021-10-11T18:24:33.000+0000",

…

Once that is obtained, add it to the group_roles object in the script.

group_roles = [
    # Google Workspace GROUP Name, XIQ group ID
    ("PPSK Users Group”, "769490635824436"),
    ("Google Workspace GROUP Name", "XIQ User Group ID")
]

If needed, continue to gather other Google Workspace Group Names and XIQ User Group IDs. Enter them in the same format, with each set enclosed in parentheses. All but the last one should be followed by a coma.

group_roles = [
    # Google Workspace GROUP Name, XIQ group ID
    ("d1ba206a-0213-4ae1-95de-0778b366b778", "769490635824438"),
    ("e7274e17-d57f-4cb3-b3e5-fe7231f952e5", "769490635824436"),
    ("27a8d84c-f4c4-41e2-a84e-9abdac7c0c13", "769490635824395")
]

## XIQ Network Policy ID

Each XIQ Network Policy will be assigned a unique ID when created. The easiest way to get the ID is to select the Network Policy in the XIQ GUI. When you choose the Network Policy, the ID is the long number in the URL, and the Policy Name is directly under the Policy Details.

You can also get the Network Policy ID from Swagger for all Network Policies with PCG configured.

Return to the swagger page, scroll to the Configuration – User Management section, and find the /pcg/key-based GET request. If the Network Policy is configured to use PCG, it will be listed in the response. The policy id and policy name are included in this response.

Response Body

[

{

"id": 1059916324374423,

"create_time": "2021-12-13T15:37:55.000+0000",

"update_time": "2022-01-06T20:29:15.000+0000",

"org_id": 0,

"policy_id": 1059916324374274,

"policy_name": "JB_Lab",

"ssid_name": "PCG_Test",

"enabled": true,

"users": [

{

# Running the Script:

Python3 XIQ-GSuite-PPSK-Sync.py

To run the script, open the terminal (PowerShell for Windows) to the location of the script and run:

chmod +x XIQ-GSuite-PPSK-Sync.py

You can also make the script executable by running

./XIQ-GSuite-PPSK-Sync.py

Then you can run the script by typing

The script will print to the screen how many PPSK users and Google Workspace users were parsed. If any users are in the list of Google Workspace users and not in the list of PPSK users, an API call will be made to create the PPSK user. The script will print on the screen for each user it successfully creates.

successfully created PPSK user Tim Smith

If there are any users in the list of PPSK users that are not in the list of Google Workspace users or disabled Google Workspace users, an API call will be made to delete the PPSK user. The script will print on the screen for each user it successfully deletes. – This will be the email address of the user.

User user0200@example.com - 769490635839948 was successfully deleted.

The Google Workspace user must have an email address assigned, or the PPSK user will not be created. A message will print on the screen.

User Ruby Smith doesn't have an email set and will not be created in xiq

## Log File

Upon running the script, a log file will be created named XIQ-GSuite-PPSK-Sync.log. Additional runs of the script will append to this log file. This file will contain the same information that prints to the screen and any error received when making the API calls. This is an excellent place to look if issues are seen.

# Scheduling Script to Run

## Mac & Linux-based Systems

A Cron job can be set up to run the script at a specified interval automatically. Ideally, this could be set for every 8, 12, or 24 hours, ensuring that the Google Workspace and PPSK user groups would stay in sync. The script can also be run manually between those times if a user needs to be added or removed immediately.

### Setting up a Cron Job

Open and edit the crontab and configure the job with the arrangement for the command you want to run. From the terminal window, enter the following command.

crontab -e

There are three parts to a cron job configuration.

#### Cron Job Time Format

Part 1 of the cron job

The first five characters, a b c d e, represent the job's time, date, and repetition.

a – Minute (0-59) 
b – Hour (0-23)

c – Day (0-31)

d – Month (0-12) – 0=None and 12 = December

e – Day of the Week (0-7) – 0=Sunday and 7=Sunday

- An asterisk (*) stands for all values. Use this operator to keep tasks running during all months, or all days of the week.

- A forward-slash (/) is used to divide a value into steps. (*/2 would be every other value, */3 would be every third, */10 would be every tenth, etc.)

The time format would look like this to set the Cron job to run every 8 hours.

0 */8 * * *

The time format would look like this to set the Cron job to run every night at midnight.

0 0 * * *

#### Cron Job Script and Script Location

Part 2 of the cron job

The next part is where you enter the script you want to run and its location.

python3 /home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync.py

You can make the script executable, so you don’t have to type python3 before entering the script. Instead, you will enter a period before the location.

chmod +x /home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync.py

./home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync.py

#### Cron Job Output and Job Completion

Optional Part 3 of the cron job

The last part is an optional part that specifies where the output and completion of the script should go. If not set, the cron will send an email to the owner of the crontab file.

To avoid filling up the inbox on the server, it is recommended to have something set for the output. This can be set to append to a file.

>> /home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync-Output.txt

> /dev/null 2>&1

Or can it be set just to turn off the email output

#### Cron Job Command Example

The command should be entered in a single line and saved in the crontab file.

Every 8 hours turning off the output

0 */8 * * * python3 /home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync.py > /dev/null 2>&1

Every 12 hours with saved output

0 */12 * * * ./home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync.py >> /home/admin/documents/scripts/XIQ-GSuite-PPSK-Sync-Output.txt

## Windows based Systems

A Windows task schedule can be set up to run the script at a specified interval automatically. Ideally, this could be set for every 8, 12, or 24 hours, ensuring the Google Workspace and PPSK user groups would stay in sync. The script can also be executed manually between those times if a user needs to be added or removed immediately.

### Setting up Windows Task Scheduler

Open Control Panel > System and Security > Administrative Tools > Task Scheduler

Selected ‘Create basic task….’

Give your task a name like ‘Google Workspace-PPSK-Sync’ and click ‘Next.’

Leave the Trigger set to daily and click ‘Next’ – We will come back and adjust this.

Click ‘Next’ leaving recur every one day

Select ‘Start a program and click ‘Next.’

#### Start a Program

For the Program/script: section, enter the path of your python.exe file.

sys.executable

Import sys

The location of your python.exe file depends on how it was installed. An easy way to find this location is to open Windows PowerShell and enter python3. This will open the Python interpreter. In the interpreter, enter                      then.

exit()

This will output the location of the python.exe file. Enter               to exit the interpreter.

Enter the full path of the python.exe file in the Program/Script: field.

Enter the script's name in the Add arguments (optional) field.

XIQ-GSuite-PPSK-Sync.py

Enter the script's location in the Start in the (optional) field.

C:\user\your_python_project_path

#### Editing the Time

Once the task is saved, open the Task Scheduler Library folder, and find the newly created Google Workspace-PPSK-Sync task. Click on it to open, select the Trigger tab, and edit the Daily trigger. Here you can set what time you want it to run.

If you want the script to run every 8 or 12 hours, check the box next to ‘Repeat task every:’ and enter ‘8 hours’ or ’12 hours’

# Troubleshooting:

## Log File

The XIQ-GSuite-PPSK-sync.log file is an excellent place to look for potential issues. This log file will update whenever the script is run, manually or on a schedule.

### Invalid XIQ token

2021-11-05 16:58:27: root - ERROR - Error retrieving PPSK users from XIQ - HTTP Status Code: 401

2021-11-05 16:58:27: root - WARNING -       {'error_code': 'AuthInvalidToken', 'error_id': 'cda656a5157d4c87a5143252aad71bff', 'error_message': 'Unable to read JSON value: ?[\x19???\x14?M??'}

Check token using Swagger - remember that if you generate a specific token, it may only have access to the user APIs.

### Invalid XIQ token format

2021-11-08 13:56:36: root - ERROR - Error retrieving PPSK users from XIQ - HTTP Status Code: 401

2021-11-08 13:56:36: root - WARNING -       {'error_code': 'AuthInvalidToken', 'error_id': '555d1ce9f67b40ef83caf4a89ca92b04', 'error_message': 'JWT strings must contain exactly 2 period characters. Found: 0'}

This may mean you are trying to use XIQ Username and Password but did not comment on line 28. Add a # in front of line 28. Or the token wasn’t entered correctly

### Expired XIQ token – Code 401 & JWT expired

2021-11-08 14:14:16: root - ERROR - Error retrieving PPSK users from XIQ - HTTP Status Code: 401

2021-11-08 14:14:16: root - WARNING -       {'error_code': 'AuthTokenExpired', 'error_id': '78d8b818a03940dd8d4accfc1b3ffb7e', 'error_message': 'JWT expired at 2021-11-08T19:14:11Z.

The XIQ token has expired. Generate a new token using Swagger.

### Invalid XIQ Username/password – Code 401

2021-11-08 13:55:15: root - ERROR - Error getting access token - HTTP Status Code: 401

2021-11-08 13:55:15: root - WARNING -       <Response [401]>

Check username and password for XIQ. – I recommend using XIQ Token

### Failed Authentication for Google Workspace API

2022-09-22 16:23:24: root - ERROR - Google Workspace_token.json was not found. Please run the 'Gsuite_setup.py' script to authorize the Google Workspace API and receive an API token.

Check that the gsuite_token.json file is in the correct location.

2022-09-22 16:25:44: root - ERROR - Failed to refresh Google Workspace Token with - ('deleted_client: The OAuth client was deleted.', {'error': 'deleted_client', 'error_description': 'The OAuth client was deleted.'})

The OAuth Credentials have been deleted. Please recreate the OAuth Credentials

2022-09-22 16:33:06: root - ERROR - Request had invalid authentication credentials. Expected OAuth 2 access token, login cookie or another valid authentication credential.

The token may be corrupt. Delete gsuite_token.json and rerun the gsuite_setup.py script.

### Invalid or Wrong Group Name entered

2022-09-22 16:35:18: root - ERROR - Group 'PPSK Users' was not found in domain bmatlabs.com

Check the group name for the Google Workspace user group

### XIQ User Failed to Create – Code 400

2021-11-08 14:19:35: root - INFO - Successfully parsed 0 XIQ users

2021-11-08 14:19:36: root - INFO - Successfully parsed 4 LDAP users

2021-11-08 14:19:36: root - ERROR - Error adding PPSK user Tim Smith - HTTP Status Code: 400

2021-11-08 14:19:36: root - WARNING -       {'error_code': 'UNKNOWN', 'error_id': None, 'error_message': 'UNKNOWN'}

2021-11-08 14:19:36: root - ERROR - failed to create Bauer Smith: Error adding PPSK user Tim Smith - HTTP Status Code: 400

There are a couple of possibilities when you see this error. As you can see above, there were 0 XIQ users parsed, and the user failed to create. In this instance, the XIQ User Group ID was incorrect. If the XIQ users parsed was 0 and you have configured users in the user group, check the user group ID in the group_roles list on Lines 30-34

The other thing that could cause this Error/Warning is if the username already exists in XIQ PPSK users within a different user group.

### XIQ Timeout Error

2021-10-08 12:43:05: root - ERROR - Failed retrieving PPSK users from XIQ - HTTP Status Code: 504

2021-10-08 12:43:05: root - WARNING -       <Response [504]>

An HTTP Status Code: 504 is a timeout from XIQ. If XIQ cannot respond to the API call within 60 secs, it will send these 504 errors. The script will need to be rerun with no changes.

## Gsuite_test.py

This script was written to help troubleshoot issues with collecting data from Google Workspace. The variables are the same in this script, except that the Google Workspace group name should be added to the gs_group_name variable instead of in the group_roles list.

This script will test the Google Workspace API. If there are any errors, they will print to the screen. Otherwise, the collected data from the Google Workspace cloud will print on the screen. If empty [] brackets print, check the Group Name and validate that it is correct.

Using this output, you can check whether the user is disabled in Google Workspace. You can validate that there is an email set for the user. And overall, check that information is being returned.

Kayce Smith {'accountEnabled': False, 'email': 'kayce@bmatlabs.com', 'username': 'kayce@bmatlabs.com'}

Sheng Smith {'accountEnabled': True, 'email': 'sheng@bmatlabs.com', 'username': 'sheng@bmatlabs.com'}

Tim Smith {'accountEnabled': True, 'email': 'tim@bmatlabs.com', 'username': 'tim@bmatlabs.com'}